HNC CANOPEN CONFIGURATION

Contents

1.Download and install driver	2

2. Wiring	2

2.1. Connect CAN	3

2.2. Connect CN1	3

3. Configuration	4

3.1. Reset encoder	5

3.2. Display all parameter	5

3.3. Set max speed during torque control	6

3.4. set encoder to maximum resolution	7

3.5. Enable CanOpen	9

3.6. Config filter	9

3.7. enable Force stop	10

3.8. disable prohibit for testing	11

3.9. Save paramters to Servo Driver	11

3.10. Restart servo	12

# 1.Download and install driver

http://www.hncelectric.com/en_download.aspx?cid=&category_id=0&page=2

there is a document too

# 2. Wiring

LAN Connector

DB25 connector

CN1 Connector

## 2.1. Connect CAN

Use LAN cable (20NM use CN3, 30 NM use CN4, can test which port can use)

| CN3 or CN4 | Description | DB25 |
| --- | --- | --- |
| 1 | CAN H | 18 |
| 2 | CAN L | 6 |
| 3 | GND | 14 |

## 2.2. Connect CN1

Use at least 7 wires cable

| CN1 | Description | DB25 |
| --- | --- | --- |
| 13 | 24V | 1 |
| 14 | Servo On | 2 |
| 15 | Force stop (E-Stop) | 15 |
| 16 | P-OT | 8 |
| 17 | N-OT | 8 |
| 23 | Z+ | 13 |
| 24 | Z- | 25 |

# 3. Configuration

- Use mini usb to USB connect servo driver to PC

- Open iwatch -> choose serial port (not port 1) -> seach -> choose device -> connect

- Click C/E to choose English

## 3.1. Reset encoder

If driver has alarm A.810 reset encoder.

- Click “excecute”  power off driver   power on again.

## 3.2. Display all parameter

PN00B-> set “operator parameter display selection” too “display all parameters”

## 3.3. Set max speed during torque control

PN407 (here I set 150 rpm, you can choose from 120-200, depend on you :D)

## 3.4. set encoder to maximum resolution

PN20E and PN210

Config encoder as single turn

## 3.5. Enable CanOpen

PN011: Hex 2 choose 800kbps, hex3 enable can open

## 3.6. Config filter

Enable filter and cutoff freq to 200hz

## 3.7. enable Force stop

## 3.8. disable prohibit for testing

After testing work change like this

## 3.9. Save paramters to Servo Driver

## 3.10. Restart servo